DISHA MVP

Digital Infrastructure for Spacecraft Handling and Analytics

Complete Architecture and Module Reference

Detailed explanation of every module, backend architecture, API layer,

frontend components, and how all systems connect together.

Orbtrix Space Technologies

March 2026

1. System Overview	5

1.1 Technology Stack	5

1.2 Directory Structure	5

2. How the Backend Architecture Works	7

2.1 Application Startup (main.py)	7

2.2 The Simulation Loop (telemetry_loop)	7

3. Backend Core Modules — Detailed Explanation	9

3.1 MissionState (mission_state.py)	9

3.1.1 Constructor (__init__)	9

3.1.2 tick(dt_seconds)	9

3.1.3 Subsystem Update Methods	9

3.1.4 Other Methods	9

3.2 Flight Dynamics (flight_dynamics.py)	11

3.2.1 Time Functions	11

3.2.2 Coordinate Transforms	11

3.2.3 Orbit Propagation (HPOP with J2)	11

3.2.4 Eclipse Model	11

3.2.5 Visibility and Orbital Elements	11

3.3 TLE Manager (tle_manager.py)	13

3.3.1 How TLE Loading Works	13

3.3.2 Methods	13

3.4 Power Module (power_module.py)	14

3.4.1 predict_power()	14

3.4.2 project_power()	14

3.4.3 check_power_feasibility()	14

3.5 FDIR Engine (fdir_engine.py)	15

3.5.1 Architecture	15

3.5.2 Default Rules	15

3.5.3 Evaluation Logic	15

3.6 Constraint Engine (constraint_engine.py)	16

3.6.1 How Risk Score Works	16

3.6.2 Margin System	16

3.6.3 Recommended Mode	16

3.7 Ground Station Module (ground_stations.py)	17

3.7.1 Preset Networks	17

3.7.2 Contact Checking (check_contact_now)	17

3.7.3 Pass Prediction (GroundStationPassPredictor)	17

3.8 Autonomy Manager (autonomy_manager.py)	18

3.8.1 Three-Mode State Machine	18

3.8.2 Hysteresis	18

3.8.3 Objective Selection	18

3.8.4 Confidence Computation	18

3.8.5 Subsystem Coupling	18

3.8.6 Feasibility Validation	18

3.8.7 Operator Override	18

3.9 Command Engine (command_engine.py)	19

3.9.1 Command Templates	19

3.9.2 Workflow	19

3.10 Mission Planner (mission_planner.py)	20

3.10.1 Greedy Scheduler	20

3.10.2 Resource Models	20

3.10.3 Feasibility Scoring	20

3.10.4 Conflict Detection	20

3.11 Telemetry Manager (telemetry_manager.py)	21

3.11.1 ConnectionManager	21

3.11.2 build_telemetry_frame()	21

3.12 Telemetry Recorder (telemetry_recorder.py)	22

3.12.1 CSV Recording	22

3.12.2 JSON Recording	22

4. Data Models and Constants	23

4.1 Pydantic Schemas (schemas.py)	23

4.2 Constants (constants.py)	23

4.3 Configuration Loader (config.py)	23

5. API Layer — How Backend Connects to Each API Endpoint	24

5.1 Core Routes (api/core.py)	24

5.2 TLE Routes (api/tle.py)	24

5.3 Flight Routes (api/flight.py)	24

5.4 FDIR Routes (api/fdir.py)	24

5.5 Planning Routes (api/planning.py)	25

5.6 Intelligence Routes (api/intelligence.py)	25

5.7 WebSocket Route (api/websocket.py)	26

5.8 Recorder Routes (api/recorder.py)	26

6. Frontend Architecture — How the Frontend Connects to Everything	27

6.1 Application Entry Point	27

6.2 The WebSocket Hook (useWebSocket.js)	27

6.3 The API Service (api.js)	27

6.4 Routing and Pages	27

6.5 Key Components and What They Connect To	27

6.5.1 Header (Header.jsx)	27

6.5.2 CesiumGlobe (CesiumGlobe.jsx)	28

6.5.3 GroundTrack2D (GroundTrack2D.jsx)	28

6.5.4 Telemetry Sidebar (Telemetry.jsx)	28

6.5.5 AutonomyPanel (AutonomyPanel.jsx)	28

6.5.6 FDIRPanel (FDIRPanel.jsx)	28

6.5.7 SchedulePanel (SchedulePanel.jsx)	28

6.5.8 ControlStrip (ControlStrip.jsx)	28

6.5.9 FlightDashboard (FlightDashboard.jsx)	28

7. Complete Data Flow — From Simulation to Screen	30

7.1 Real-Time Telemetry Flow (Every Second)	30

7.2 REST API Flow (On Demand)	30

7.3 TLE Loading Flow	30

7.4 Mission Planning Flow	30

8. Configuration Reference (satellite_config.json)	31

# 1. System Overview

DISHA MVP is a deterministic satellite mission control platform. It simulates and manages a Low Earth Orbit satellite in real-time. There is no machine learning or AI anywhere in the system. Every decision is rule-based, traceable, and explainable. The system is split into two main parts: a Python backend that runs the actual satellite simulation and logic, and a React frontend that displays everything visually to the operator.

The backend is a FastAPI application running on Python 3.11+. It creates a single in-memory satellite object (called the digital twin) and advances its state by one second every tick in a background loop. Every tick, the backend propagates the orbit, checks if the satellite is in contact with any ground station, runs fault detection rules, evaluates constraints, runs the autonomy manager, and broadcasts the full telemetry frame to all connected frontend clients over WebSocket.

The frontend is a React 19 + Vite 7 application. It connects to the backend via a single WebSocket at ws://127.0.0.1:8000/ws/telemetry and receives a telemetry frame every second. It also makes REST API calls (via axios) for things like loading TLEs, generating mission plans, viewing orbital elements, and controlling the autonomy system. The frontend renders a 3D globe (CesiumJS), a 2D ground track (Leaflet), telemetry gauges, autonomy panels, FDIR alerts, and a mission scheduling interface.

## 1.1 Technology Stack

| Layer | Technologies |
| --- | --- |
| Backend Runtime | Python 3.11+, FastAPI, Uvicorn (ASGI server) |
| Backend Libraries | NumPy (math/arrays), sgp4 (TLE propagation), httpx (HTTP client), Pydantic v2 (validation) |
| Frontend Runtime | React 19, Vite 7 (build tool), Node.js |
| Frontend Libraries | CesiumJS 1.117+ (3D globe), Leaflet 1.9+ (2D map), Recharts (charts), Lucide React (icons), axios (HTTP) |
| Communication | WebSocket (real-time telemetry at 1 Hz), REST/JSON (commands, queries) |
| Configuration | JSON config file (satellite_config.json) loaded once at startup |

## 1.2 Directory Structure

| Path | Purpose |
| --- | --- |
| backend/main.py | Application entry point. Creates all global objects, registers routes, starts the 1 Hz simulation loop. |
| backend/core/ | All core simulation modules: orbit propagation, power, FDIR, autonomy, mission planning, etc. |
| backend/api/ | FastAPI route files. Each file groups related endpoints. Routes call into core/ modules. |
| backend/models/ | Pydantic schemas (schemas.py), WGS84 constants (constants.py), config loader (config.py). |
| config/ | satellite_config.json — all satellite parameters, thresholds, FDIR rules, constraint rules. |
| frontend/src/App.jsx | Root component. Sets up routing, WebSocket connection, and passes data to pages. |
| frontend/src/pages/ | Four dashboard pages: Control, Flight, FDIR, Schedule. |
| frontend/src/components/ | Reusable UI components: CesiumGlobe, GroundTrack2D, Telemetry, AutonomyPanel, etc. |
| frontend/src/services/api.js | Axios API client. Every REST call the frontend makes is defined here. |
| frontend/src/hooks/useWebSocket.js | Custom React hook for WebSocket connection, auto-reconnect, and state management. |

# 2. How the Backend Architecture Works

## 2.1 Application Startup (main.py)

When you run the backend, main.py does everything in order. First, it calls load_config() which reads config/satellite_config.json and caches it as a global singleton dictionary. If the file does not exist, a hardcoded default config is used instead. Then it creates all the global objects that make up the running system:

| Object | What It Does |
| --- | --- |
| satellite = MissionState() | The digital twin. Holds all satellite state: position, velocity, battery, temperature, storage, comms, attitude, contact status, etc. |
| tle_manager = TLEManager() | Manages TLE data. Can fetch TLE from CelesTrak by NORAD ID and propagate position using SGP4. |
| fdir_engine = FDIREngine() | Fault Detection, Isolation, and Recovery. Evaluates configurable rules against telemetry every tick. |
| ws_manager = ConnectionManager() | WebSocket connection manager. Tracks all connected frontend clients and broadcasts to them. |
| pass_predictor = GroundStationPassPredictor() | Predicts when the satellite will pass over each ground station in the next 24 hours. |
| command_engine = CommandEngine() | Generates telecommand sequences from mission plans. Tracks approval workflow. |
| autonomy_manager = AutonomyManager() | Three-mode autonomy state machine (AUTONOMOUS / GUARDED / SAFE). Makes objective decisions. |
| telemetry_recorder = TelemetryRecorder() | Auto-records every telemetry frame to CSV. Supports optional JSON snapshots. |
| intelligence_cache = {} | A dictionary that caches constraint and autonomy results so REST endpoints can serve them without recomputation. |

After creating all objects, satellite.tle_manager is set to point to the tle_manager object so the satellite can use SGP4 propagation when a TLE is loaded. Then main.py registers all API route modules (core, tle, flight, fdir, planning, intelligence, websocket, recorder) onto the FastAPI app. CORS middleware is added with allow_origins=['*'] so any frontend can connect.

## 2.2 The Simulation Loop (telemetry_loop)

The heart of the backend is an async function called telemetry_loop() that runs as a background task. It starts when the app boots (via the lifespan context manager) and runs forever, executing once per second. Here is exactly what happens every tick, in order:

Step 1: Advance the simulation clock by 1 second.

The satellite.tick(dt_seconds=1.0) method is called. Inside tick(), if a TLE is loaded, the satellite uses SGP4 to compute its new position and velocity at the new time. If no TLE is loaded, it falls back to HPOP (High-Precision Orbit Propagator) using RK4 numerical integration with J2 perturbation. Either way, the satellite gets a new ECI position and velocity. Then tick() converts ECI to ECEF (using GMST rotation), then ECEF to LLA (latitude, longitude, altitude using iterative WGS84). Then it checks for eclipse using a cylindrical shadow model. Finally it calls five subsystem update methods: _update_power, _update_thermal, _update_comms, _update_storage, _update_attitude.

Step 2: Check ground station contact.

The function check_contact_now() is called with the satellite's current ECI position and time. This function gets all active ground stations, converts the satellite position to ECEF, and for each station computes the elevation angle. If the satellite is above a station's minimum elevation (default 5 degrees), the satellite is in contact with that station. The function returns the station with the highest elevation. Then satellite.update_contact() is called which sets the contact flags (in_contact, contact_station, contact_elevation_deg) and refreshes the comms subsystem accordingly. During contact, SNR is computed from elevation angle (higher elevation = better signal). During blackout (no contact), SNR is zero and link status is NO_CONTACT.

Step 3: Run FDIR evaluation.

The fdir_engine.evaluate(raw_state) method is called. It loops through all configured FDIR rules and checks each one against the current telemetry values. Rules use operators like <, >, ==, != to compare parameter values against thresholds. When a rule triggers, an FDIRAlert object is created. The engine is self-clearing: alerts are automatically removed when the condition returns to normal. The engine also skips SNR rules during blackout because SNR=0 is expected when not in contact.

Step 4: Run constraint evaluation and autonomy manager.

evaluate_constraints() computes a weighted risk score from all constraint violations. Each constraint rule has a weight, and the total risk is the sum of triggered weights (capped at 1.0). It also computes margins for each subsystem (POWER, THERMAL_PANEL, THERMAL_BATTERY, COMMS, STORAGE, ORBIT) showing how far each parameter is from its safe operating range. The result is cached in intelligence_cache['constraints'].

Then autonomy_manager.evaluate() is called with the telemetry, constraint result, upcoming approved tasks, and power predictions. The autonomy manager selects an objective using configurable priority rules, transitions between modes using hysteresis (separate enter/exit thresholds), computes confidence scores, evaluates subsystem coupling effects, checks feasibility, and generates structured decision objects. The result is cached in intelligence_cache['autonomy'].

Step 5: Broadcast telemetry.

build_telemetry_frame() constructs a structured telemetry frame with nested objects (position, velocity, power, thermal, comms, attitude, storage) plus flat legacy fields for backward compatibility. During contact, the frame source is 'LIVE'. During blackout, the source is 'PREDICTED'. If contact was just acquired, the onboard buffer is dumped and sent as a 'buffer_dump' message first. Every frame is also recorded to CSV by the telemetry_recorder.

# 3. Backend Core Modules — Detailed Explanation

## 3.1 MissionState (mission_state.py)

This is the central digital twin. It holds every piece of information about the satellite's current state. There is only one instance of this class, created at startup. All other modules read from and write to this object.

### 3.1.1 Constructor (__init__)

When MissionState is created, it reads the satellite_config.json through get_config(). It extracts four sections: orbit, power, thermal, storage, and comms. From these, it initializes all state variables:

Orbital state: position (ECI, km) defaults to [7000, 0, 0], velocity (ECI, km/s) defaults to [0, 7.5, 0]. These represent a roughly 622 km circular orbit. Geographic position (lat, lon, alt) starts at zero and is computed each tick.

Power subsystem: battery_capacity_wh (default 500 Wh), current_battery_wh (starts full), bus_voltage (12V nominal), solar_panel_current (1.5A nominal), solar_array_output_w (18W), base_load_w (3W constant draw).

Thermal subsystem: component_temp (25C), battery_temp (22C), heater_active (False).

Comms subsystem: snr_db (15 dB nominal), link_status ('NOMINAL'), data_rate_kbps (256).

Attitude: mode ('NADIR'), pointing_error (0.1 deg), angular_rate (0.01 deg/s).

Storage: capacity is 1 TB (1048576 MB), used starts at 0.

Contact state: in_contact (False), contact_station (None), blackout_duration_sec (0). The onboard buffer can hold up to 5400 snapshots (90 minutes at 1 Hz).

It also holds references for FDIR alerts, autonomy state, risk score, command history, and mission schedule. The tle_manager reference is set to None initially and assigned later in main.py.

### 3.1.2 tick(dt_seconds)

This is the most important method. It advances the entire satellite simulation by dt_seconds (default 1.0). First it propagates the orbit: if a TLE is loaded (tle_manager.satrec is not None), it uses SGP4 via tle_manager.propagate_at(). Otherwise it uses the custom HPOP propagator via rk4_step(). Then it converts ECI to ECEF to LLA, computes altitude, checks eclipse, and calls five private update methods for power, thermal, comms, storage, and attitude.

### 3.1.3 Subsystem Update Methods

_update_power(dt): During eclipse, solar current is 0 and net power is negative (base load drain). During sunlit, solar current is 1.5A with small random noise, and net power is solar output minus base load. Battery is clamped between 0 and capacity.

_update_thermal(dt): During eclipse, temperature drifts downward. If component_temp drops below -20C, the heater activates. During sunlit, temperature drifts upward. Temperature is clamped to [-50, 100] for components and [-10, 55] for battery.

_update_comms(dt): During contact, SNR is computed from elevation angle: base_snr = 8 + (elevation/90)*12, with small random noise. If SNR >= 12, link is NOMINAL at 256 kbps. If SNR < 12, link is DEGRADED at 64 kbps. During blackout, SNR is 0, link is NO_CONTACT, data rate is 0.

_update_attitude(dt): Simplified model. Pointing error stays near 0.1 deg with small random noise. Angular rate stays near 0.01 deg/s.

### 3.1.4 Other Methods

get_state(): Returns a flat dictionary with every state variable. Used by the API and telemetry broadcast. Contains both structured field names (battery_wh, battery_pct) and legacy aliases (battery_soc, solar_current).

update_contact(in_contact, station, elevation): Sets contact flags and refreshes comms. Returns True if contact was just acquired (transition from blackout to contact), which triggers buffer dump.

buffer_telemetry(snapshot): Stores a state snapshot in the onboard buffer during blackout. If buffer exceeds 5400 entries, oldest is dropped.

dump_buffer(): Returns all buffered telemetry and clears the buffer. Called when contact is acquired.

update_state(power_cost_wh, data_cost_gb): Deducts power and adds storage usage from task execution.

reset(): Recreates the entire object from scratch by calling __init__().

## 3.2 Flight Dynamics (flight_dynamics.py)

This module contains pure mathematical functions with no internal state. It handles all orbital mechanics, coordinate transforms, eclipse detection, and visibility computation.

### 3.2.1 Time Functions

get_julian_date(dt): Converts a Python datetime (UTC) to Julian Date using the standard astronomical formula. This is needed by GMST computation and sun position.

get_gmst(dt): Computes Greenwich Mean Sidereal Time in radians using the IAU formula. GMST tells you how much the Earth has rotated, which is needed to convert from the inertial ECI frame to the Earth-fixed ECEF frame.

get_sun_position(dt): Computes the approximate position of the Sun in the ECI frame using the low-precision solar position algorithm. It computes the mean longitude and anomaly of the Sun, the equation of center, the ecliptic longitude, and applies the obliquity of the ecliptic to get XYZ coordinates in ECI. The result is in km.

### 3.2.2 Coordinate Transforms

eci_to_ecef(r_eci, dt): Rotates an ECI position vector to the ECEF frame using a Z-axis rotation by the GMST angle. This accounts for the Earth's rotation. r_ecef = Rz(GMST) * r_eci.

ecef_to_lla(r_ecef): Converts ECEF [x, y, z] in km to geodetic latitude, longitude, and altitude. Uses an iterative method (up to 20 iterations) with WGS84 ellipsoid parameters (flattening = 1/298.257, eccentricity squared). Converges to 1e-6 radian tolerance.

lla_to_ecef(lat_deg, lon_deg, alt_km): The reverse transform. Converts geodetic coordinates back to ECEF. Uses the WGS84 normal radius N = a / sqrt(1 - e^2 * sin^2(lat)).

### 3.2.3 Orbit Propagation (HPOP with J2)

get_j2_acceleration(position): Computes gravitational acceleration with J2 perturbation. J2 accounts for Earth's equatorial bulge, which causes orbital precession. The formulas multiply the standard two-body gravity by correction factors involving (z/r)^2 and (R_earth/r)^2. This is the most important perturbation for LEO satellites.

rk4_step(state, dt): A single step of the 4th-order Runge-Kutta integrator. The state vector is [x, y, z, vx, vy, vz]. It computes four slope estimates (k1 through k4) and combines them with the standard RK4 formula. This gives much better accuracy than simple Euler integration.

propagate_orbit(initial_state, duration_seconds, step_size): Propagates an orbit forward by the given duration using rk4_step at each time step. Returns a list of {time_offset, eci_state} dictionaries. This is used by the power prediction module and pass predictor.

### 3.2.4 Eclipse Model

is_in_eclipse(position_eci, dt): Implements a cylindrical shadow model. It computes the sun direction vector, projects the satellite position onto this direction. If the projection is negative (satellite is on the dark side of Earth) and the perpendicular distance from the sun-Earth line is less than Earth's radius, the satellite is in eclipse.

predict_eclipse_simple(position_eci): A faster version that assumes the Sun is always along the +X axis. Used by the power prediction module for quick eclipse estimation over 90-minute windows.

### 3.2.5 Visibility and Orbital Elements

compute_elevation(sat_ecef, station_lat, station_lon, station_alt_km): Computes the elevation angle of the satellite as seen from a ground station. Converts station position to ECEF, computes the range vector from station to satellite, and takes the dot product with the station's local up vector to get the sine of the elevation angle.

is_visible(sat_ecef, station_lat, station_lon, min_elevation_deg, station_alt_km): Returns True if the computed elevation exceeds the minimum elevation threshold (default 5 degrees). Returns a tuple of (bool, elevation_deg).

state_to_keplerian(position, velocity, mu): Converts a position-velocity state vector to the six classical Keplerian orbital elements: semi-major axis, eccentricity, inclination, RAAN, argument of periapsis, and true anomaly. Also computes the orbital period. This uses standard vector cross products and dot products from astrodynamics.

check_feasibility(request, mission_state): Propagates the orbit and checks if the satellite will be visible from a requested target location during the given time window. Returns whether the task is feasible and the access windows.

## 3.3 TLE Manager (tle_manager.py)

This module handles Two-Line Element sets from CelesTrak. A TLE is a standardized format that describes a satellite's orbit. The TLEManager can fetch a TLE by NORAD catalog number and use the SGP4 algorithm (from the sgp4 library) to compute position and velocity at any given time.

### 3.3.1 How TLE Loading Works

When the operator enters a NORAD ID in the frontend, the frontend calls POST /tle/load. The API handler calls tle_manager.fetch_tle(norad_id). This method does an HTTP GET to CelesTrak's GP API (celestrak.org). The response contains three lines: satellite name, TLE line 1, and TLE line 2. These are parsed, and Satrec.twoline2rv() creates an SGP4 satellite record. After loading, the MissionState is reset, position and velocity are computed at the current UTC time using SGP4, and the FDIR engine is cleared.

### 3.3.2 Methods

fetch_tle(norad_id): Async method that fetches TLE from CelesTrak. Uses httpx sync client in a thread executor because httpx async has issues on Windows. Parses the three-line response, creates SGP4 satellite record. Returns TLE metadata dict.

propagate_at(dt): Propagates to the given datetime using SGP4. Converts datetime to Julian date parts (jd, fr), calls satrec.sgp4(), returns (position_km, velocity_km_s) as lists. Raises RuntimeError if SGP4 returns a non-zero error code.

get_tle_info(): Returns metadata: NORAD ID, satellite name, TLE lines, loaded status, and epoch age in hours.

clear(): Resets all TLE data to None/empty.

## 3.4 Power Module (power_module.py)

This module predicts battery state of charge over time and checks whether the satellite has enough power to execute tasks.

### 3.4.1 predict_power()

This is the main prediction function. It propagates the orbit for 90 minutes at 1-minute intervals and simulates the battery level at each step. At each step, it checks if the satellite will be in eclipse (using predict_eclipse_simple). In sunlit, solar panels generate 18W. In eclipse, generation is 0. The base load is always 3W. If there are scheduled tasks, extra power draw is added based on task type: IMAGING = 8W, DOWNLINK = 12W, MANOEUVRE = 6W, CONTACT = 10W. The function also tracks storage changes (IMAGING generates 0.4 GB/min, DOWNLINK offloads 1.5 GB/min). Returns prediction_points (array of SOC% over time), min_soc_pct, power_margin_wh, and storage predictions.

### 3.4.2 project_power()

Simulates 100 minutes forward at 30-second intervals to find the next eclipse transition. Returns the current battery %, the projected battery % at the next eclipse entry, the projected battery % at the end of one orbit, and a power warning flag if either projection drops below 25%.

### 3.4.3 check_power_feasibility()

Quick power budget check for a single task. Given the current battery Wh, task power draw in watts, and task duration in seconds, it computes the energy cost (W * s / 3600 = Wh) and whether the resulting battery level stays above the warning threshold (default 20%).

## 3.5 FDIR Engine (fdir_engine.py)

FDIR stands for Fault Detection, Isolation, and Recovery. This module evaluates a set of configurable rules against current telemetry and generates alerts when thresholds are breached.

### 3.5.1 Architecture

The FDIREngine class holds a list of rules (loaded from config), a dictionary of active alerts (keyed by rule_id), and a deque of alert history (last 200). Alerts are self-clearing: when a condition returns to normal, the alert is automatically removed. No operator intervention is needed to clear alerts.

### 3.5.2 Default Rules

| Rule ID | Parameter | Op | Threshold | Corrective Action |
| --- | --- | --- | --- | --- |
| BATT_CRITICAL | battery_soc | < | 20% | Switch to SAFE mode. Disable non-essential loads. |
| BATT_LOW | battery_soc | < | 40% | Reduce payload duty cycle. Defer low-priority downlinks. |
| TEMP_PANEL_HIGH | component_temp | > | 85C | Adjust attitude to reduce solar exposure. |
| TEMP_PANEL_LOW | component_temp | < | -40C | Check thermal control heater status. |
| TEMP_BATT_HIGH | battery_temp | > | 45C | Reduce charge rate. Enable battery heater. |
| TEMP_BATT_LOW | battery_temp | < | 0C | Enable battery heater. |
| STORAGE_HIGH | storage_pct | > | 90% | Prioritize downlink during next ground station pass. |
| SNR_LOW | snr | < | 8 dB | Increase transmit power. Re-point antenna. |
| SNR_CRITICAL | snr | < | 5 dB | Switch to backup antenna. Abort data transfer. |
| POINTING_ERROR | pointing_error | > | 2.0 deg | Re-initialize ADCS. Check reaction wheel status. |
| ALT_LOW | altitude | < | 200 km | Evaluate orbit-raising maneuver. |

### 3.5.3 Evaluation Logic

The evaluate() method iterates through all rules. For each rule, it maps the parameter name to the telemetry field (using a flexible mapping that tries multiple field name conventions). It then compares the value against the threshold using the specified operator (<, >, ==, !=). SNR rules are skipped when the satellite is not in contact (since SNR=0 is expected during blackout). When a rule triggers and wasn't already active, a new FDIRAlert object is created with full metadata (rule_id, severity, parameter, current value, threshold, timestamp, corrective action). Alerts that were active but are no longer triggered are removed (self-clearing).

## 3.6 Constraint Engine (constraint_engine.py)

The constraint engine is separate from FDIR. While FDIR generates alerts for individual parameter violations, the constraint engine computes an overall risk score and per-subsystem margin status.

### 3.6.1 How Risk Score Works

Each constraint rule has a weight (0 to 1). When a constraint fires, its weight contributes to the total risk score. Rules are deduplicated by category (only the highest-weight rule per category counts). The total risk is the sum of deduplicated weights, clamped to [0, 1]. For example, if POWER (weight 0.35) and COMMS (weight 0.30) are both violated, the risk score is 0.65.

### 3.6.2 Margin System

The constraint engine defines six margin categories, each with three operating ranges (nominal, warning, critical):

| Category | Parameter | Nominal | Warning | Critical |
| --- | --- | --- | --- | --- |
| POWER | battery_soc | 40-100% | 25-100% | 15-100% |
| THERMAL_PANEL | component_temp | -20 to 60C | -30 to 75C | -40 to 85C |
| THERMAL_BATTERY | battery_temp | 5 to 35C | 0 to 40C | -5 to 45C |
| COMMS | snr | 10-50 dB | 8-50 dB | 5-50 dB |
| STORAGE | storage_pct | 0-80% | 0-90% | 0-95% |
| ORBIT | altitude | 300-800 km | 250-1000 km | 200-1200 km |

Each margin shows a status (NOMINAL, WARNING, CRITICAL, VIOLATION) and a margin_pct showing how far the value is from the critical boundary.

### 3.6.3 Recommended Mode

Based on the risk score and autonomy config thresholds: if risk >= 0.6 or any CRITICAL violation exists, recommended mode is SAFE. If risk >= 0.3, recommended mode is GUARDED. Otherwise, AUTONOMOUS.

## 3.7 Ground Station Module (ground_stations.py)

This module manages ground station networks and predicts contact windows.

### 3.7.1 Preset Networks

Five predefined networks are available: ISRO (8 ISTRAC stations across India and Indian Ocean region), NASA (6 DSN and tracking stations), ESA (6 ESTRACK stations worldwide), KSAT (6 stations including SvalSat in Svalbard), and GLOBAL (8 stations selected for maximum worldwide coverage). The default is ISRO. Operators can switch networks, add custom stations, or remove stations via the API.

### 3.7.2 Contact Checking (check_contact_now)

This function is called every tick. It takes the satellite's ECI position and current time, converts to ECEF, then for each active ground station, computes the elevation angle using compute_elevation(). If the elevation exceeds the station's minimum elevation threshold (5 degrees), the station is a candidate. The station with the highest elevation is selected as the contact station. Returns a dict with in_contact (bool), station name, and elevation.

### 3.7.3 Pass Prediction (GroundStationPassPredictor)

The compute_passes() method propagates the orbit for a given duration (default 24 hours) at 10-second intervals. For each time step and each station, it checks if the satellite is visible. It tracks when visibility starts (AOS — Acquisition of Signal) and when it ends (LOS — Loss of Signal). Passes shorter than 30 seconds are discarded. Returns a list of pass objects sorted by AOS time, each containing station name, coordinates, AOS/LOS times, duration, and maximum elevation.

## 3.8 Autonomy Manager (autonomy_manager.py)

This is the intelligence layer of the system. It makes autonomous decisions about what the satellite should be doing based on current conditions. It is completely deterministic — every decision can be traced back to specific conditions and rules.

### 3.8.1 Three-Mode State Machine

The autonomy manager operates in one of three modes:

AUTONOMOUS: Normal operations. The satellite can execute any scheduled task, make imaging passes, perform downlinks, and operate with full capability. This is the default mode.

GUARDED: Cautious operations. The satellite operates but with restrictions. Non-essential tasks may be deferred. Entered when risk score exceeds 0.3 (guarded_enter threshold).

SAFE: Emergency mode. Only essential operations (power management, thermal protection). Entered when risk score exceeds 0.6 (safe_enter threshold) or when any CRITICAL constraint violation exists.

### 3.8.2 Hysteresis

Mode transitions use separate enter and exit thresholds to prevent oscillation. Escalation (toward SAFE) is always immediate because safety is urgent. De-escalation (toward AUTONOMOUS) requires the new conditions to persist for a minimum dwell time (default 30 seconds). For example: to enter GUARDED, risk must exceed 0.3. To exit GUARDED back to AUTONOMOUS, risk must drop below 0.2 (guarded_exit) and stay there for 30 seconds.

### 3.8.3 Objective Selection

The autonomy manager selects an objective from configurable priority rules (loaded from satellite_config.json under the 'autonomy.objective_rules' key). Each rule has conditions (parameter comparisons using lt, gt, eq, neq, lte, gte operators), allowed modes, and a priority. Rules are evaluated in priority order. The first rule whose conditions all match (AND logic) and whose mode is allowed becomes the active objective. If no rule matches, the fallback is 'Nominal Orbit Maintenance'.

### 3.8.4 Confidence Computation

Confidence is not just 1 - risk_score. It is computed from four factors: risk score impact (weight 0.4), number of violations (weight 0.2), critical violation count (weight 0.2), and margin health (weight 0.2). The resulting score is classified as HIGH (>= 0.8), MEDIUM (>= 0.5), or LOW.

### 3.8.5 Subsystem Coupling

The autonomy manager models interactions between subsystems. Four coupling effects are detected: POWER_PAYLOAD_CONFLICT (low battery + high storage), COMMS_STORAGE_CONFLICT (degraded comms + high storage), ECLIPSE_POWER_STRESS (eclipse + low battery), THERMAL_POWER_CONFLICT (thermal warning + low battery). Each effect includes a description, affected subsystems, severity, and recommendation.

### 3.8.6 Feasibility Validation

Before selecting objectives, the manager checks five feasibility dimensions: power (battery above minimum), communication (SNR above minimum), storage (below maximum), attitude (pointing error within capability), and power margin (net power sufficient for payload).

### 3.8.7 Operator Override

Operators can force any mode via POST /intelligence/override. Override bypasses automatic mode selection until released via POST /intelligence/override/release. All overrides are logged with operator name, reason, and duration.

## 3.9 Command Engine (command_engine.py)

This module generates telecommand sequences from mission plan tasks and manages the approval workflow.

### 3.9.1 Command Templates

Four task types have predefined telecommand sequences:

| Task Type | Command Sequence (in order) |
| --- | --- |
| IMAGING | ADCS_SLEW (slew to nadir) -> PAYLOAD_ON -> IMAGING_START -> IMAGING_STOP -> PAYLOAD_OFF |
| DOWNLINK | ADCS_SLEW (slew to station) -> COMMS_TX_ON -> DOWNLINK_START -> DOWNLINK_STOP -> COMMS_TX_OFF |
| MANOEUVRE | ADCS_SLEW (slew to burn attitude) -> THRUSTER_ARM -> BURN_START -> BURN_STOP -> THRUSTER_SAFE |
| CONTACT | ADCS_SLEW (slew to station) -> COMMS_TX_ON -> CONTACT_START -> CONTACT_STOP -> COMMS_TX_OFF |

### 3.9.2 Workflow

generate_sequence(plan_details, plan_id): Takes a list of tasks from a mission plan and creates a command sequence. Each task generates 5 commands from its template. Commands are spaced 5 seconds apart within each task. The sequence starts in PENDING status.

approve_sequence(sequence_id, operator): Approves a command sequence for dispatch. Changes status to APPROVED and records the operator name and timestamp. All individual commands are also marked APPROVED.

Operators can also send ad-hoc commands via POST /commands/send (SAFE MODE, PAYLOAD ON/OFF, ATTITUDE NADIR/SUN, TX HIGH/LOW, HEATER ON/OFF).

## 3.10 Mission Planner (mission_planner.py)

This module schedules satellite tasks using a greedy algorithm and computes feasibility scores.

### 3.10.1 Greedy Scheduler

generate_mission_plan() takes a list of user requests (each with target coordinates, priority, and time window). Requests are sorted by priority (highest first). For each request, the planner checks its feasible windows (computed by check_feasibility in flight_dynamics.py) and selects the first window that does not overlap with already-scheduled tasks. If no non-overlapping window exists, the request is rejected. Each scheduled task gets a power cost (Wh) and data cost (GB) based on its type and duration.

### 3.10.2 Resource Models

| Task Type | Power (W) | Data Rate (GB/s) | Notes |
| --- | --- | --- | --- |
| IMAGING | 15.0 | 0.5 | High power, generates data |
| DOWNLINK | 20.0 | 0.0 | Highest power, offloads data |
| MANOEUVRE | 12.0 | 0.0 | Moderate power, no data |
| CONTACT | 10.0 | 0.0 | Communication only |

### 3.10.3 Feasibility Scoring

compute_feasibility() scores each scheduled task on three factors: power margin (50% weight, based on projected battery after task), contact availability (25% weight, whether a ground pass exists within 30 minutes), and sunlit status (25% weight, whether the satellite is currently in sunlight). Score >= 0.75 is LOW risk, >= 0.5 is MEDIUM risk, below is HIGH risk.

### 3.10.4 Conflict Detection

detect_conflicts() checks all task pairs for time overlap, mode conflicts (imaging and downlink cannot run simultaneously), and cumulative power deficits (projected battery dropping below 20%).

## 3.11 Telemetry Manager (telemetry_manager.py)

This module has two parts: the WebSocket ConnectionManager and the telemetry frame builder.

### 3.11.1 ConnectionManager

Manages WebSocket connections for real-time telemetry broadcast. connect(ws) accepts and registers a new client. disconnect(ws) removes a client. broadcast(data) sends a JSON message to all connected clients. If a send fails, that client is auto-disconnected. The client_count property returns the number of active connections.

### 3.11.2 build_telemetry_frame()

Constructs the telemetry frame that gets sent via WebSocket every second. It takes the raw state dict from MissionState.get_state(), FDIR alerts, and a source tag ('LIVE', 'PREDICTED', or 'BUFFERED'). The frame contains structured nested objects (position, velocity, power, thermal, comms, attitude, storage) for the new frontend, plus flat legacy fields for backward compatibility. Every frame also includes contact metadata (in_contact, contact_station, elevation, blackout duration) and the satellite name.

## 3.12 Telemetry Recorder (telemetry_recorder.py)

Auto-records every telemetry frame to CSV and supports optional JSON snapshot recording.

### 3.12.1 CSV Recording

CSV recording starts automatically when the recorder is created. A new CSV file is created in the telemetry_logs/ directory with a timestamped filename. Every time record() is called (once per tick), the telemetry frame is flattened into 30 columns (timestamp, position, velocity, power, thermal, comms, attitude, storage, contact, alert count) and written as a CSV row. The file is flushed every 60 frames to prevent data loss.

### 3.12.2 JSON Recording

JSON recording is manual (start/stop). When started via start_json(), frames are buffered in memory. When stopped via stop_json(), all buffered frames are written to a JSON file with metadata (recorded timestamp, total frame count). This is useful for capturing specific events.

# 4. Data Models and Constants

## 4.1 Pydantic Schemas (schemas.py)

All request and response models are defined using Pydantic v2 BaseModel. The telemetry frame has nested models: PositionData (lat, lon, alt), VelocityData (speed, vx, vy, vz), PowerData (battery_soc, battery_voltage, current_draw, solar_current, in_eclipse), ThermalData (component_temp, heater_active), CommsData (link_status, snr, data_rate, nearest_station), AttitudeData (mode, pointing_error, angular_rate), StorageData (used_mb, capacity_mb, storage_pct), FDIRAlertModel (rule_id, severity, parameter, current_value, threshold, timestamp, corrective_action). The top-level TelemetryFrame model nests all of these.

Other models: UserRequest (target coordinates, priority, time window), Task (task_id, action, start/end time, power/data cost), MissionPlan (request_id, is_feasible, schedule list), TLELoadRequest (norad_id), OrbitalElements (six Keplerian elements + period), GroundStationPass (station, coordinates, AOS/LOS, duration, max elevation), Telecommand (command details), CommandSequence (list of commands with approval status), ScheduleRequest (list of target dicts).

## 4.2 Constants (constants.py)

WGS84 Earth constants used throughout the system: MU_EARTH = 398600.4418 km^3/s^2 (gravitational parameter), EARTH_RADIUS_KM = 6378.137 km (equatorial radius), J2_COEFF = 1.08262668e-3 (oblateness), EARTH_ROTATION_RATE = 7.292115e-5 rad/s, WGS84_FLATTENING = 1/298.257223563, WGS84_E_SQ = 2f - f^2. Also: SPEED_OF_LIGHT = 299792458 m/s, SOLAR_CONSTANT = 1361 W/m^2. Default satellite configuration values are also defined here.

## 4.3 Configuration Loader (config.py)

Singleton config loader. load_config() reads satellite_config.json from the config/ directory. If the file does not exist, it returns a hardcoded default config. The config is cached in a module-level variable _config. get_config() returns the cached config (calling load_config() on first access). The config contains all satellite parameters (orbit, power, thermal, storage, comms, attitude), FDIR rules, constraint rules, ground station definitions, and autonomy settings.

# 5. API Layer — How Backend Connects to Each API Endpoint

The backend has 8 API route files, each registered as a FastAPI router in main.py. Every route file has a get_deps() function that imports global objects from main.py at call time (to avoid circular imports). Here is every endpoint, what backend module it calls, and what it returns:

## 5.1 Core Routes (api/core.py)

| Endpoint | Backend Module | What It Returns |
| --- | --- | --- |
| GET / | satellite, tle_manager, ws_manager | System status: name, version, uptime, WS client count, TLE loaded flag |
| GET /satellite-status | satellite.get_state() | Full satellite state snapshot (all subsystems) |
| POST /reset | reset_state() in main.py | Resets all global objects to initial state |

## 5.2 TLE Routes (api/tle.py)

| Endpoint | Backend Module | What It Returns |
| --- | --- | --- |
| POST /tle/load | tle_manager.fetch_tle() + satellite.reset() | Fetches TLE from CelesTrak, resets satellite, propagates new position via SGP4 |
| GET /tle/current | tle_manager.get_tle_info() | Current TLE metadata: NORAD ID, satellite name, lines, epoch age |

## 5.3 Flight Routes (api/flight.py)

| Endpoint | Backend Module | What It Returns |
| --- | --- | --- |
| GET /orbit/prediction | tle_manager or propagate_orbit() | 90-minute orbit prediction: list of lat/lon/alt points at 60s intervals |
| GET /flight/orbital-elements | state_to_keplerian() | Current Keplerian elements: a, e, i, RAAN, omega, true anomaly, period |
| GET /flight/passes | pass_predictor.compute_passes() | 24-hour ground station pass predictions for all active stations |
| GET /flight/ground-stations | get_ground_stations() | List of active ground stations with coordinates |
| GET /flight/ground-networks | get_available_networks() | Available preset networks with station counts |
| POST /flight/ground-stations/set | set_ground_stations() | Switch to a preset network (ISRO, NASA, ESA, KSAT, GLOBAL) |
| POST /flight/ground-stations/add | add_custom_station() | Add a custom ground station by name and coordinates |
| POST /flight/ground-stations/remove | remove_station() | Remove a station by name |

## 5.4 FDIR Routes (api/fdir.py)

| Endpoint | Backend Module | What It Returns |
| --- | --- | --- |
| GET /fdir/alerts | fdir_engine.get_active_alerts() + .get_history() | Currently active alerts + full alert history |
| GET /fdir/status | fdir_engine.get_status() | Engine status: rules count, active alerts count, total alerts |
| GET /fdir/summary | fdir_engine.get_summary() | Risk status (NOMINAL/WARNING/CRITICAL), counts, auto actions today |

## 5.5 Planning Routes (api/planning.py)

| Endpoint | Backend Module | What It Returns |
| --- | --- | --- |
| POST /generate-plan | check_feasibility() + generate_mission_plan() + command_engine.generate_sequence() | Full mission plan: scheduled tasks, feasibility scores, conflicts, command sequence ID |
| GET /power/prediction | predict_power() | 90-minute SOC prediction curve with eclipse/task overlay |
| GET /commands | command_engine.get_all_sequences() | All command sequences with status |
| GET /commands/log | command_engine.get_log() | Full command log (generate, approve, ad-hoc) |
| GET /commands/{id} | command_engine.get_sequence() | Single command sequence by ID |
| POST /commands/{id}/approve | command_engine.approve_sequence() | Approve a sequence for dispatch |
| POST /commands/send | Direct satellite state modification | Execute ad-hoc command (SAFE MODE, PAYLOAD ON/OFF, etc.) |

## 5.6 Intelligence Routes (api/intelligence.py)

| Endpoint | Backend Module | What It Returns |
| --- | --- | --- |
| GET /intelligence/autonomy | intelligence_cache['autonomy'] | Full autonomy status: mode, objective, confidence, decisions, coupling, feasibility |
| GET /intelligence/constraints | intelligence_cache['constraints'] | Constraint evaluation: risk score, violations, margins, recommended mode |
| GET /intelligence/margins | From constraint cache | Per-subsystem margin status with percentages |
| GET /intelligence/power-projection | project_power() | Battery projections at next eclipse and orbit end |
| GET /intelligence/decisions | autonomy_manager.get_decisions_log() | Full decision log with structured decision objects |
| GET /intelligence/decisions/latest | autonomy_manager.get_latest_decisions() | Last N decisions |
| GET /intelligence/feasibility | autonomy_manager.get_status().feasibility | Current feasibility assessment for all dimensions |
| GET /intelligence/coupling | autonomy_manager.get_status().coupling_effects | Active subsystem coupling effects |
| POST /intelligence/override | autonomy_manager.set_mode() | Force autonomy mode (operator override) |
| POST /intelligence/override/release | autonomy_manager.release_override() | Release override, resume automatic selection |

## 5.7 WebSocket Route (api/websocket.py)

WS /ws/telemetry: A WebSocket endpoint. When a client connects, ws_manager.connect() accepts it. The server never reads from the client (but the client must keep the connection alive). The 1 Hz telemetry_loop in main.py broadcasts frames to all connected clients via ws_manager.broadcast().

## 5.8 Recorder Routes (api/recorder.py)

| Endpoint | Backend Module | What It Returns |
| --- | --- | --- |
| GET /recorder/status | telemetry_recorder.status() | CSV file path, frame count, JSON recording state |
| POST /recorder/json/start | telemetry_recorder.start_json() | Starts JSON frame buffering |
| POST /recorder/json/stop | telemetry_recorder.stop_json() | Stops JSON recording, saves to file, returns path |
| GET /recorder/download/csv | FileResponse on CSV file | Downloads the current session CSV telemetry log |
| GET /recorder/download/json | FileResponse on latest JSON | Downloads the most recent JSON snapshot file |

# 6. Frontend Architecture — How the Frontend Connects to Everything

## 6.1 Application Entry Point

main.jsx renders the App component into the DOM root. App.jsx wraps everything in a ThemeProvider (dark/light theme context) and BrowserRouter (React Router). The AppContent component is where the real work happens. It creates a single WebSocket connection to ws://127.0.0.1:8000/ws/telemetry using the useWebSocket custom hook, and passes the resulting telemetry data down to all child components.

## 6.2 The WebSocket Hook (useWebSocket.js)

This is the most critical piece of frontend infrastructure. It creates a WebSocket connection and manages all real-time state. The hook returns: telemetry (latest frame), alerts (list of FDIR alerts), connected (boolean), contactState (in_contact, source, station, elevation, blackout duration), bufferDump (frames received when contact is acquired), telemetryHistory (last 1800 frames = 30 minutes at 1 Hz), clearAlerts(), clearBufferDump().

The WebSocket receives three types of messages: 'telemetry' (every 1 second, updates telemetry and contact state, adds to history), 'buffer_dump' (when contact is acquired, contains stored blackout frames), and 'tle_loaded' (when a new satellite is loaded, clears all old state). Auto-reconnect is built in: if the connection drops, it retries after 2 seconds. History is updated every 10 frames to avoid excessive re-renders.

## 6.3 The API Service (api.js)

All REST API calls are made through the api object exported from services/api.js. It uses axios with a base URL of http://127.0.0.1:8000 and a 30-second timeout. There are 30+ methods covering every API endpoint. Each method wraps the call in try/catch and returns null on error. This means the frontend never crashes from API errors — it just gets null and can handle it gracefully.

## 6.4 Routing and Pages

The frontend has four routes, each mapping to a page component:

| Path | Page | What It Shows |
| --- | --- | --- |
| / | ControlDashboard | Main mission control view: 3D globe, 2D ground track, autonomy panel, telemetry sidebar, event log, command terminal |
| /flight | FlightDashboard | Orbital mechanics view: orbit prediction chart, power SOC prediction, Keplerian elements, ground station passes, TLE loader, ground network selector |
| /fdir | FDIRDashboard | Fault monitoring: subsystem health sparklines, FDIR alert cards, margin gauges, autonomy status, constraint violations, decision timeline |
| /schedule | ScheduleDashboard | Mission planning: target map picker, priority selector, plan generator, feasibility scores, conflict viewer, command sequence approval, Gantt timeline |

## 6.5 Key Components and What They Connect To

### 6.5.1 Header (Header.jsx)

Navigation bar at the top. Shows satellite name, system health status (online/offline based on WebSocket connection), critical alert count, eclipse indicator (sun/moon icon from telemetry.in_eclipse), battery level, view navigation buttons (Control, Flight, FDIR, Schedule), reset button, theme toggle, and UI scale control. The reset button calls api.resetSatellite() which hits POST /reset.

### 6.5.2 CesiumGlobe (CesiumGlobe.jsx)

3D globe visualization using CesiumJS. Every telemetry frame, it updates the satellite position (converting lat/lon/alt to Cesium Cartesian3). It draws the orbit track by fetching /orbit/prediction and plotting the points as a polyline. It shows ground station markers (fetched from /flight/ground-stations) with colored icons. The globe shows day/night terminator. Satellite tracking (camera follows satellite) can be toggled. The eclipse state changes the satellite dot color. Orbit track is refreshed periodically.

### 6.5.3 GroundTrack2D (GroundTrack2D.jsx)

2D Mercator map using Leaflet. Shows the same information as CesiumGlobe but on a flat map. Satellite position updates from telemetry. Orbit ground track is fetched from /orbit/prediction. Ground stations are fetched from /flight/ground-stations and shown with colored emoji markers (color-coded by network: ISRO=teal, ESA=purple, NASA=pink, KSAT=gold, Custom=cyan).

### 6.5.4 Telemetry Sidebar (Telemetry.jsx)

Displays all subsystem values from the live telemetry frame. Organized into sections: Position (lat, lon, alt), Power (battery %, voltage, solar current, eclipse status), Thermal (panel temp, battery temp, heater), Comms (link status, SNR, data rate, nearest station), Attitude (mode, pointing error, angular rate), Storage (used GB, percentage). Each value has a sparkline showing the last 60 data points. Values change color based on warning thresholds. Also shows contact state (in contact/blackout, station name, elevation). Polls /intelligence/constraints for margin status.

### 6.5.5 AutonomyPanel (AutonomyPanel.jsx)

Center panel on the Control Dashboard. Three stacked sections: A) Autonomy Status Card showing current mode (with color coding), objective, confidence level, risk score, last decision with trigger constraints and violations; B) Next Action Timeline showing upcoming actions with time offsets; C) Rejected Actions showing objectives that were infeasible or not allowed in the current mode. Polls /intelligence/autonomy and /intelligence/constraints every 2 seconds.

### 6.5.6 FDIRPanel (FDIRPanel.jsx)

Full FDIR monitoring dashboard. Left column shows subsystem health with sparklines (battery, temperature, SNR, voltage, solar current, storage — tracking last 60 values). Center shows active FDIR alert cards with severity color coding and corrective actions. Right column shows margin gauges for all six margin categories, autonomy mode/objective/confidence, recent decisions with trigger constraints, and constraint violation cards. Polls /fdir/summary, /intelligence/autonomy, /intelligence/constraints, /intelligence/decisions/latest every 2 seconds.

### 6.5.7 SchedulePanel (SchedulePanel.jsx)

Mission planning interface. Left section has a target picker with 10 Indian cities as presets (or enter custom lat/lon), priority selector (CRITICAL/HIGH/MEDIUM/LOW), and a list of queued targets. Generate Plan button calls POST /generate-plan. Center section shows the plan results with feasibility scores (color-coded risk level), conflicts, and a command sequence with APPROVE button (calls POST /commands/{id}/approve). Right section shows power SOC prediction chart (from /power/prediction) with eclipse shading and task load overlay. Bottom shows a Gantt-style timeline (ScheduleTimeline component) of scheduled tasks.

### 6.5.8 ControlStrip (ControlStrip.jsx)

Bottom strip of the Control Dashboard. Contains three sub-components: EventLog (displays FDIR alerts, contact events, and buffer dumps in a scrolling log format), CommandTerminal (interactive terminal where operators can type or select satellite commands like PAYLOAD ON, SAFE MODE, etc. — sends to POST /commands/send), and PassCountdown (shows next ground pass time and current contact status).

### 6.5.9 FlightDashboard (FlightDashboard.jsx)

The Flight page is the most chart-heavy page. It contains: TLE Loader (input NORAD ID, calls POST /tle/load), Orbital Elements display (from /flight/orbital-elements), Power SOC prediction area chart (from /power/prediction), Power Projection summary (from /intelligence/power-projection), Orbit Prediction visualization, Ground Station Pass timeline (from /flight/passes), and Ground Network selector (from /flight/ground-networks, with buttons to switch via /flight/ground-stations/set). Each chart uses Recharts (AreaChart, ReferenceLine, Tooltip). The page uses Spark components that show small inline sparklines tracking real-time parameter history.

# 7. Complete Data Flow — From Simulation to Screen

## 7.1 Real-Time Telemetry Flow (Every Second)

1. telemetry_loop() in main.py calls satellite.tick(1.0) which advances orbit, updates all subsystems.

2. check_contact_now() determines if satellite is above any ground station.

3. satellite.update_contact() sets contact flags and updates comms subsystem.

4. fdir_engine.evaluate() checks all rules against the new state, generating or clearing alerts.

5. evaluate_constraints() computes risk score and margins from constraint rules. Result is cached.

6. autonomy_manager.evaluate() selects objective, transitions mode if needed, builds decision objects. Result is cached.

7. build_telemetry_frame() constructs the frame with all data + metadata.

8. telemetry_recorder.record() writes the frame to CSV (and JSON if recording).

9. ws_manager.broadcast() sends the frame as JSON to all connected WebSocket clients.

10. In the frontend, useWebSocket's onmessage handler receives the frame, updates telemetry state, updates contact state, adds to history, and triggers React re-renders.

11. All components that receive telemetry as a prop re-render with the new data. CesiumGlobe moves the satellite marker. Telemetry sidebar updates gauges. AutonomyPanel shows latest status.

## 7.2 REST API Flow (On Demand)

When the frontend needs data that is not in the WebSocket stream (like orbital elements, pass predictions, or power projections), it makes an axios GET/POST request via the api service. The request goes to the backend, which either returns cached data (intelligence endpoints) or computes it on the fly (orbit prediction, pass prediction). The response is processed by the component that requested it.

## 7.3 TLE Loading Flow

1. Operator enters NORAD ID in the Flight Dashboard TLE loader. 2. Frontend calls api.loadTLE(noradId) which hits POST /tle/load. 3. Backend fetches TLE from CelesTrak, creates SGP4 satellite record. 4. Backend resets satellite, propagates new position, clears FDIR. 5. Backend broadcasts a 'tle_loaded' WebSocket message. 6. Frontend useWebSocket detects 'tle_loaded', clears all old state (telemetry, alerts, history, buffer). 7. Next tick, new satellite telemetry starts flowing.

## 7.4 Mission Planning Flow

1. Operator adds targets on the Schedule Dashboard (lat/lon coordinates with priority). 2. Operator clicks Generate Plan. Frontend calls api.generatePlan(targets) which hits POST /generate-plan. 3. Backend runs check_feasibility() for each target (propagates orbit, checks visibility). 4. Feasible requests go to generate_mission_plan() (greedy scheduler). 5. Command sequences are generated by command_engine. 6. Backend returns plan details, feasibility scores, conflicts, and command sequence ID. 7. Frontend displays results with approve button. 8. Operator clicks Approve. Frontend calls api.approveCommandSequence(id) which hits POST /commands/{id}/approve. 9. Backend marks sequence as approved. Approved tasks are factored into the next power prediction.

# 8. Configuration Reference (satellite_config.json)

The satellite_config.json file controls all system behavior. Here are the key sections and what they control:

| Section | Key Parameters | Purpose |
| --- | --- | --- |
| satellite | name, norad_id | Satellite identity |
| orbit | initial_position_eci_km, initial_velocity_eci_km_s | Starting orbital state when no TLE is loaded |
| power | battery_capacity_wh (500), solar_array_output_w (18), base_load_w (3), task_loads_w | Power budget: generation, consumption, task costs |
| thermal | panel_temp_nominal_c (25), battery_temp_nominal_c (22), warning/critical thresholds | Thermal operating ranges and alarm thresholds |
| storage | capacity_mb (1048576 = 1 TB) | Onboard storage capacity |
| comms | snr_nominal_db (15), snr_warning_db (8), snr_critical_db (5) | Communication link parameters |
| attitude | default_mode (NADIR), pointing_error_limit_deg (2.0), angular_rate_limit_deg_s (1.0) | Attitude control parameters |
| fdir_rules | Array of 11 rules with rule_id, parameter, operator, threshold, severity, corrective_action | All FDIR rules evaluated every tick |
| constraint_rules | Array of 12 rules with parameter, threshold, weight, category, operator | Constraint rules for risk score computation |
| ground_stations | Array of 8 ISRO ISTRAC stations with name, lat, lon, alt_m, min_elevation_deg | Default ground station network |
| autonomy | mode_thresholds, min_dwell_time_sec, objective_rules, feasibility limits, coupling config | Autonomy manager behavior: mode transitions, objective selection, feasibility checks |