DISHA MVP

Digital Infrastructure for Spacecraft Handling and Analytics

Detailed Low-Level Module Reference

Every Class, Method, Parameter, and Data Structure

Version 1.0 | March 2026 | Orbtrix

# 1. Project Overview

DISHA (Digital Infrastructure for Spacecraft Handling and Analytics) is a deterministic, rule-based satellite mission control platform. It provides real-time orbit propagation via HPOP (J2 perturbation with RK4 integration) and SGP4, autonomous decision-making through a 3-mode state machine, fault detection with self-clearing alerts, and contact-aware telemetry with onboard buffering during blackout periods.

The backend runs on Python 3.11+ with FastAPI and exposes REST endpoints plus a WebSocket at 1 Hz. The frontend is a React 19 + Vite 7 application with CesiumJS 3D globe and Leaflet 2D maps. All autonomy decisions are deterministic and fully traceable; no machine learning is used.

### Technology Stack

| Layer | Technologies |
| --- | --- |
| Backend | Python 3.11+, FastAPI 0.110+, Uvicorn, NumPy, Pydantic v2, sgp4, httpx |
| Frontend | React 19, Vite 7, CesiumJS 1.117+, Leaflet 1.9+, Recharts 2.x, Lucide React, React Router DOM 6 |
| Deployment | Docker, Docker Compose, multi-container (backend + frontend + Nginx) |
| Real-time | WebSocket at 1 Hz telemetry push; asyncio background simulation loop |
| Data Storage | In-memory state, CSV auto-recording, optional JSON snapshots |

# 2. Directory Structure

| Path | Description |
| --- | --- |
| backend/ | Python server: FastAPI app, simulation core, models |
| backend/core/ | 12 simulation engine modules (orbit, power, FDIR, autonomy, etc.) |
| backend/api/ | 8 FastAPI router modules (REST + WebSocket endpoints) |
| backend/models/ | Pydantic schemas, physical constants, JSON config loader |
| backend/main.py | Application entry point, simulation loop, global state initialization |
| frontend/src/components/ | React UI components (CesiumGlobe, Telemetry, FDIR, etc.) |
| frontend/src/pages/ | Dashboard pages (Control, Flight, FDIR, Schedule) |
| frontend/src/hooks/ | Custom hooks (useWebSocket, useTheme) |
| frontend/src/services/ | HTTP client (api.js) and CesiumJS helpers |
| config/satellite_config.json | Master satellite configuration (11 sections) |
| telemetry_logs/ | Auto-recorded CSV files and optional JSON snapshots |

# 3. Backend Models Package

## 3.1 constants.py — Physical Constants

File: backend/models/constants.py. Defines WGS84 Earth parameters and physical constants used across all simulation modules.

| Attribute | Type / Default | Description |
| --- | --- | --- |
| MU_EARTH | 3.986004418e5 km³/s² | Earth gravitational parameter (GM) |
| EARTH_RADIUS_KM | 6378.137 km | WGS84 equatorial radius |
| EARTH_RADIUS_M | 6378137.0 m | Equatorial radius in meters |
| EARTH_ROTATION_RATE | 7.292115e-5 rad/s | Earth sidereal rotation rate |
| J2_COEFF | 1.08262668e-3 | J2 oblateness perturbation coefficient |
| WGS84_FLATTENING | 1/298.257223563 | WGS84 ellipsoid flattening |
| WGS84_E_SQ | ~0.00669438 | WGS84 eccentricity squared (derived) |
| SPEED_OF_LIGHT | 299792458.0 m/s | Speed of light in vacuum |
| SOLAR_CONSTANT | 1361.0 W/m² | Solar irradiance at 1 AU |
| deg2rad / rad2deg | numpy derived | Conversion helpers |

Also defines DEFAULT_ORBIT (position [7000,0,0] km, velocity [0,7.5,0] km/s), DEFAULT_POWER (500 Wh battery, 18W solar, 3W base load, 12V bus), DEFAULT_THERMAL thresholds, DEFAULT_STORAGE (1024 GB), and DEFAULT_COMMS (SNR nominal 15 dB, warning 8 dB, critical 5 dB).

## 3.2 config.py — Configuration Loader

File: backend/models/config.py. Singleton JSON config loader. Maintains a global _config dict initialized on first call to load_config() or get_config().

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| load_config(path=None) | path: str ¦ None (override file path) | Loads JSON from config/satellite_config.json. Returns dict. Caches globally. |
| get_config() | (none) | Returns cached config, loads if not yet loaded. |
| _get_default_config() | (none) | Returns hardcoded fallback dict with all 11 config sections. |

Default config file path is computed relative to the module: ../../config/satellite_config.json. Config sections: satellite, orbit, power, thermal, storage, comms, attitude, fdir_rules (11 rules), constraint_rules (12 rules), ground_stations (8 ISRO stations). Full autonomy config is loaded from JSON if present.

## 3.3 schemas.py — Pydantic Models

File: backend/models/schemas.py. All Pydantic v2 BaseModel classes for API request/response contracts.

### Telemetry Models

| Attribute | Type / Default | Description |
| --- | --- | --- |
| PositionData | latitude, longitude, altitude | All float. Geographic position. |
| VelocityData | speed, vx, vy, vz | All float. Velocity components in km/s. |
| PowerData | battery_soc, battery_voltage, current_draw, solar_current, in_eclipse | float/bool. Power subsystem state. |
| ThermalData | component_temp, heater_active | float/bool. Thermal subsystem. |
| CommsData | link_status (str), snr, data_rate, nearest_station | str/float. NOMINAL¦DEGRADED¦LOST. |
| AttitudeData | mode (str), pointing_error, angular_rate | NADIR¦SUN_POINTING¦TARGET + floats. |
| StorageData | used_mb, capacity_mb, storage_pct | All float. Storage subsystem. |
| FDIRAlertModel | rule_id, severity, parameter, current_value, threshold, timestamp, corrective_action | Full alert structure. |
| TelemetryFrame | timestamp, position, velocity, power, thermal, comms, attitude, storage, fdir_alerts | Composite frame for WebSocket broadcast. |

### Task & Planning Models

| Attribute | Type / Default | Description |
| --- | --- | --- |
| TaskDefinition | name, type, priority(5), start_window, end_window, duration_seconds(300), power_draw_watts(10), requires_sunlit, requires_ground_contact | User-facing task definition. type: IMAGING¦DOWNLINK¦MANOEUVRE¦CONTACT. |
| UserRequest | request_id, target_lat, target_lon, priority, window_start, window_end, min_duration_sec(60), feasible_windows | Internal scheduling request. |
| Task | task_id, action, start_time, end_time, power_cost_wh(0), data_cost_gb(0) | Scheduled task with computed costs. |
| MissionPlan | request_id, is_feasible, reason, schedule: List[Task] | Output of mission planner. |
| Telecommand | command_id, task_id, command_type, description, parameters, scheduled_time, status(PENDING) | Single telecommand in a sequence. |
| CommandSequence | sequence_id, plan_id, commands: List[Telecommand], total_commands, created_at, approved, approved_by, approved_at | Full command sequence with approval state. |

### Other Models

| Attribute | Type / Default | Description |
| --- | --- | --- |
| AutonomyStatus | mode, current_objective, last_decision, last_decision_time, risk_score, confidence, override_active | Autonomy state snapshot. |
| TLELoadRequest | norad_id: int | Request to fetch TLE by NORAD catalog number. |
| TLEInfo | norad_id, satellite_name, tle_line1, tle_line2, loaded, epoch_age_hours | TLE metadata response. |
| OrbitalElements | semi_major_axis_km, eccentricity, inclination_deg, raan_deg, arg_periapsis_deg, true_anomaly_deg, period_min | Keplerian element snapshot. |
| GroundStationPass | station_name, latitude, longitude, country, aos_time, los_time, duration_sec, max_elevation_deg | Single pass prediction. |
| PowerPredictionPoint | time_offset_min, soc_pct, in_eclipse, solar_generation_w, load_consumption_w | Single point on SOC prediction curve. |
| PowerPrediction | prediction_points, min_soc_pct, power_margin_wh | Full 90-min power prediction. |
| ScheduleRequest | requests: list[dict] | Input for /generate-plan endpoint. |

# 4. Backend Core Modules — Detailed Reference

## 4.1 mission_state.py — Digital Twin

File: backend/core/mission_state.py. Class: MissionState. The central digital twin representing the complete satellite state. Every other module reads from or writes to this object. Called at 1 Hz by the simulation loop.

### Constructor Attributes (from config)

| Attribute | Type / Default | Description |
| --- | --- | --- |
| current_time | datetime (UTC) | Simulation clock, advanced each tick |
| position | np.array [x,y,z] km | ECI position vector (from config orbit.initial_position_eci_km, default [7000,0,0]) |
| velocity | np.array [vx,vy,vz] km/s | ECI velocity vector (default [0,7.5,0]) |
| latitude / longitude / altitude_km | float | Geographic position computed each tick via ECI->ECEF->LLA |
| battery_capacity_wh | float (500.0) | Max battery energy from power config |
| current_battery_wh | float (=capacity) | Current battery energy, decremented/incremented each tick |
| bus_voltage | float (12.0 V) | Nominal bus voltage with +/-0.3V random jitter |
| solar_panel_current | float (1.5 A) | Solar current; 0 in eclipse, 1.5+/-0.2 in sunlit |
| solar_array_output_w | float (18.0 W) | Total solar power generation |
| base_load_w | float (3.0 W) | Baseline power consumption |
| component_temp | float (25.0 C) | Panel temperature with drift model |
| battery_temp | float (22.0 C) | Battery temperature with drift model |
| heater_active | bool (False) | Heater auto-enables below -20C |
| snr_db | float (15.0 dB) | Signal-to-noise ratio; 0 during blackout |
| link_status | str (NOMINAL) | NOMINAL ¦ DEGRADED ¦ NO_CONTACT |
| data_rate_kbps | float (256.0) | Downlink data rate; 0 during blackout |
| attitude_mode | str (NADIR) | NADIR ¦ SUN_TRACKING ¦ SAFE ¦ TARGET |
| pointing_error | float (0.1 deg) | Pointing error with random jitter |
| angular_rate | float (0.01 deg/s) | Angular rate with random jitter |
| storage_capacity_mb | float (1048576) | 1024 GB in MB |
| storage_used_mb | float (0.0) | Current storage usage |
| in_eclipse | bool (False) | Eclipse state computed each tick |
| in_contact | bool (False) | Ground station contact state |
| contact_station | str ¦ None | Name of contacted station |
| contact_elevation_deg | float (0.0) | Elevation angle to contacted station |
| blackout_duration_sec | float (0.0) | Seconds since last contact lost |
| onboard_buffer | list (max 5400) | Buffered telemetry snapshots during blackout (~90 min at 1 Hz) |

### Methods

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| tick(dt_seconds=1.0) | dt_seconds: float | Advances all subsystems. Steps: (1) propagate orbit via SGP4 or RK4+J2, (2) ECI->ECEF->LLA transform, (3) eclipse check, (4) update power/thermal/comms/storage/attitude. |
| _update_power(dt) | dt: float (seconds) | Solar current=0 in eclipse, 1.5+/-0.2 in sunlit. Net power = solar - base_load. Battery clamped [0, capacity]. |
| _update_thermal(dt) | dt: float | Eclipse: temp drifts -0.5 to 0.0 per min. Sunlit: -0.2 to +0.5 per min. Heater auto-on below -20C, off above 10C. Clamped [-50, 100]C. |
| _update_comms(dt) | dt: float | In contact: SNR = 8 + (elevation/90)*12 +/- 0.5. NOMINAL if SNR>=12, DEGRADED otherwise. Blackout: SNR=0, NO_CONTACT. |
| _update_attitude(dt) | dt: float | Simplified: pointing_error = 0.1 +/- 0.05, angular_rate = 0.01 +/- 0.005. |
| get_state() | (none) | Returns dict with 40+ fields: position, velocity, lat/lon/alt, battery_pct, battery_soc, bus_voltage, component_temp, snr, link_status, attitude_mode, storage_pct, in_eclipse, in_contact, satellite_name, etc. |
| update_contact(in_contact, station, elevation) | bool, str¦None, float | Updates contact state. Returns True if contact was just acquired (for buffer dump trigger). |
| buffer_telemetry(state_snapshot) | dict | Appends to onboard_buffer. Drops oldest if > 5400. |
| dump_buffer() | (none) | Returns and clears onboard_buffer list. |
| update_state(power_cost_wh, data_cost_gb) | float, float | Deducts power and adds storage from task execution. |
| reset() | (none) | Re-initializes to constructor defaults via self.__init__(). |

## 4.2 flight_dynamics.py — Orbital Mechanics

File: backend/core/flight_dynamics.py. Pure functions module. No internal state. Provides orbit propagation, coordinate transforms, eclipse modeling, visibility computation, and orbital element extraction.

### Time Functions

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| get_julian_date(dt) | dt: datetime (UTC) | Converts UTC datetime to Julian Date using standard formula. Returns float. |
| get_gmst(dt) | dt: datetime | Computes Greenwich Mean Sidereal Time in radians using IAU formula. Uses JD->T->GMST seconds->degrees->radians. |
| get_sun_position(dt) | dt: datetime | Approximate sun position in ECI (km). Uses mean longitude L0, mean anomaly M, equation of center C, obliquity. Returns np.array [x,y,z]. |

### Coordinate Transforms

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| eci_to_ecef(r_eci, dt) | r_eci: list[3], dt: datetime | Rotates ECI vector to ECEF via GMST Z-axis rotation matrix. Returns np.ndarray. |
| ecef_to_lla(r_ecef) | r_ecef: np.ndarray[3] | Iterative WGS84 geodetic conversion (20 iterations, tol=1e-6). Returns dict {lat, lon, alt_km} in degrees. |
| lla_to_ecef(lat_deg, lon_deg, alt_km=0) | float, float, float | Geodetic to ECEF using WGS84 prime vertical radius N. Returns np.array [x,y,z] km. |

### Orbit Propagation (HPOP + J2)

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| get_j2_acceleration(position) | position: np.ndarray[3] (km) | Point-mass gravity + J2 oblateness acceleration. Returns np.array [ax,ay,az] km/s². |
| rk4_step(state, dt) | state: np.ndarray[6] [x,y,z,vx,vy,vz], dt: float (s) | Single Runge-Kutta 4th-order integration step. Returns np.ndarray[6] new state. |
| propagate_orbit(initial_state, duration_seconds, step_size=60) | dict {position, velocity}, float, float | Full trajectory over duration. Returns list of {time_offset, eci_state} dicts. |

### Eclipse & Visibility

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| is_in_eclipse(position_eci, dt) | list[3] (km), datetime | Cylindrical shadow model. Computes sun direction, projects satellite, checks if perpendicular distance < Earth radius. Returns bool. |
| predict_eclipse_simple(position_eci) | list[3] (km) | Simplified eclipse check assuming sun direction = +X axis. Used for fast prediction. Returns bool. |
| compute_elevation(sat_ecef, station_lat, station_lon, station_alt_km=0) | np.ndarray, float, float, float | Elevation angle of satellite from ground station in degrees. Uses up-vector dot product. |
| is_visible(sat_ecef, station_lat, station_lon, min_elevation_deg=5, station_alt_km=0) | np.ndarray, float, float, float, float | Returns tuple (bool, elevation_deg). |

### Orbital Elements

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| state_to_keplerian(position, velocity, mu=MU_EARTH) | list, list, float | Converts ECI state to Keplerian elements. Returns dict: semi_major_axis_km, eccentricity, inclination_deg, raan_deg, arg_periapsis_deg, true_anomaly_deg, period_min. |
| check_feasibility(request, mission_state) | UserRequest, MissionState | Propagates orbit, finds geometric access windows for target coordinates. Returns {is_feasible: bool, windows: [(start,end)...]}. |

## 4.3 tle_manager.py — TLE & SGP4

File: backend/core/tle_manager.py. Class: TLEManager. Handles TLE fetch, parse, cache, and SGP4 propagation. Uses sgp4.api library with WGS72 gravity model.

| Attribute | Type / Default | Description |
| --- | --- | --- |
| satrec | Satrec ¦ None | SGP4 satellite record object |
| tle_line1 / tle_line2 | str | Raw TLE lines |
| satellite_name | str | Satellite name from line 0 |
| norad_id | int ¦ None | NORAD catalog number |
| fetch_time | datetime ¦ None | UTC time of last fetch |

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| _fetch_tle_sync(norad_id) | norad_id: int | Sync HTTP GET to celestrak.org/NORAD/elements/gp.php. Returns raw text. Timeout: 15s. |
| fetch_tle(norad_id) | norad_id: int | Async wrapper. Parses 3-line TLE, creates Satrec via twoline2rv(). Returns get_tle_info() dict. |
| propagate_at(dt) | dt: datetime | SGP4 propagation to given datetime. Converts to JD via jday(). Returns (position_km, velocity_km_s) lists. Raises on SGP4 error. |
| get_tle_info() | (none) | Returns dict: norad_id, satellite_name, tle_line1, tle_line2, loaded, epoch_age_hours. |
| clear() | (none) | Resets all fields to None/empty. |

## 4.4 power_module.py — Power Prediction & Costing

File: backend/core/power_module.py. Stateless functions for SOC prediction, battery projection, and task power feasibility.

### Constants

| Attribute | Type / Default | Description |
| --- | --- | --- |
| TASK_DATA_RATE_GB_MIN | dict | IMAGING: 0.4, DOWNLINK: -1.5, MANOEUVRE: 0.0, CONTACT: 0.0 GB/min |
| DEFAULT_TASK_DURATION_MIN | dict | IMAGING: 5, DOWNLINK: 8, MANOEUVRE: 3, CONTACT: 10 minutes |

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| _get_power_config() | (none) | Loads solar_w (18), base_load_w (3), battery_wh (500), task_loads from config. Returns dict. |
| predict_power(mission_state, duration_minutes=90, step_minutes=1, scheduled_tasks=None) | MissionState, int, int, list¦None | 90-min SOC curve. Propagates orbit, steps through eclipse transitions. Returns: prediction_points [{time_offset_min, soc_pct, in_eclipse, solar_generation_w, load_consumption_w, task_load_w}], storage_prediction_points, min_soc_pct, power_margin_wh. |
| project_power(mission_state) | MissionState | Projects battery % at next eclipse and end of orbit (100 min, 30s steps). Returns: current_battery, current_mode (ECLIPSE¦SUNLIT), projected_next_eclipse, projected_next_orbit, time_to_next_eclipse_min, power_warning (bool), warning_reason. |
| check_power_feasibility(current_battery_wh, task_power_draw_w, task_duration_s, battery_capacity_wh=500, warning_threshold_pct=20) | floats | Checks if SOC stays above 20% threshold. Returns {feasible: bool, battery_after_pct, task_cost_wh}. |

## 4.5 fdir_engine.py — Fault Detection & Response

File: backend/core/fdir_engine.py. Classes: FDIRAlert, FDIREngine. Configurable rule evaluation with self-clearing alert lifecycle. Deterministic, no ML.

### FDIRAlert Class

| Attribute | Type / Default | Description |
| --- | --- | --- |
| rule_id | str | Rule identifier (e.g., BATT_CRITICAL) |
| severity | str | CRITICAL or WARNING |
| parameter | str | Telemetry parameter name |
| current_value | float | Actual telemetry value at trigger |
| threshold | float | Rule threshold value |
| timestamp | str (ISO) | UTC timestamp of trigger |
| corrective_action | str | Human-readable corrective action |

### Default FDIR Rules (11)

| Rule ID | Parameter | Operator | Threshold | Severity |
| --- | --- | --- | --- | --- |
| BATT_CRITICAL | battery_soc | < | 20 | CRITICAL |
| BATT_LOW | battery_soc | < | 40 | WARNING |
| TEMP_PANEL_HIGH | component_temp | > | 85 C | WARNING |
| TEMP_PANEL_LOW | component_temp | < | -40 C | WARNING |
| TEMP_BATT_HIGH | battery_temp | > | 45 C | WARNING |
| TEMP_BATT_LOW | battery_temp | < | 0 C | WARNING |
| STORAGE_HIGH | storage_pct | > | 90% | WARNING |
| SNR_LOW | snr | < | 8 dB | WARNING |
| SNR_CRITICAL | snr | < | 5 dB | CRITICAL |
| POINTING_ERROR | pointing_error | > | 2.0 deg | WARNING |
| ALT_LOW | altitude | < | 200 km | CRITICAL |

### FDIREngine Methods

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| evaluate(telemetry) | telemetry: dict | Evaluates all rules. Self-clearing: adds on trigger, removes when normal. Skips SNR rules during blackout. Returns list of active alert dicts. |
| _get_value(telemetry, param) | dict, str | Flexible field lookup with mappings: battery_soc->[battery_pct], component_temp->[panel_temp_c], etc. |
| get_active_alerts() | (none) | Returns list of active alert dicts. |
| get_history() | (none) | Returns deque(maxlen=200) as list. |
| get_status() | (none) | Returns {active, rules_enabled, rules_count, active_alerts, total_alerts}. |
| get_summary() | (none) | Returns {rules_active, last_evaluation_time, active_alerts, critical_count, warning_count, auto_actions_today, risk_status (NOMINAL¦WARNING¦CRITICAL)}. |
| reset() | (none) | Clears alerts, history, and counters. |

## 4.6 constraint_engine.py — Constraint Margins

File: backend/core/constraint_engine.py. Stateless functions for structured constraint evaluation with margins, violations, and mode recommendations.

### Margin Definitions (6 categories)

| Category | Parameter | Nominal Range | Warning Range | Critical Range |
| --- | --- | --- | --- | --- |
| POWER | battery_soc | 40-100% | 25-100% | 15-100% |
| THERMAL_PANEL | component_temp | -20 to 60 C | -30 to 75 C | -40 to 85 C |
| THERMAL_BATTERY | battery_temp | 5-35 C | 0-40 C | -5 to 45 C |
| COMMS | snr | 10-50 dB | 8-50 dB | 5-50 dB |
| STORAGE | storage_pct | 0-80% | 0-90% | 0-95% |
| ORBIT | altitude | 300-800 km | 250-1000 km | 200-1200 km |

### Functions

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| _get_value(telemetry, param) | dict, str | Flexible field name lookup via FIELD_MAPPINGS dict. |
| _compute_margin(value, margin_def) | any, dict | Computes margin status (NOMINAL¦WARNING¦CRITICAL¦VIOLATION) and margin_pct (0-100). Handles inverted parameters (storage, temp). |
| compute_margins(telemetry) | dict | Computes margins for all 6 categories. Returns dict keyed by category. |
| evaluate_constraints(telemetry_snapshot) | dict | Main function. Returns: risk_score (0-1, weighted sum of deduplicated violations), violations (list), margins (dict), recommended_mode (SAFE¦GUARDED¦AUTONOMOUS), violated_categories (list). |

Risk score computation: Each fired constraint rule has a weight. Violations are deduplicated by category (highest weight kept). Risk score = sum of deduped weights, clamped to [0,1]. Mode recommendation: risk >= 0.6 or has_critical -> SAFE; risk >= 0.3 -> GUARDED; else AUTONOMOUS.

## 4.7 ground_stations.py — Ground Station Network

File: backend/core/ground_stations.py. Manages 5 preset ground station networks plus custom stations. Provides real-time contact checking and 24-hour pass prediction.

### Preset Networks

| Network | Stations | Key Stations |
| --- | --- | --- |
| ISRO | 8 | ISTRAC Bangalore, Lucknow, Sriharikota, Thiruvananthapuram, Port Blair, Mauritius, Brunei, Biak |
| NASA | 6 | Goldstone DSN, Madrid DSN, Canberra DSN, Wallops Island, White Sands, McMurdo |
| ESA | 6 | ESTRACK Kiruna, Redu, Cebreros, Malargue, New Norcia, Kourou |
| KSAT | 6 | SvalSat Svalbard, Tromso, Grimstad, Puertollano, Mauritius, Punta Arenas |
| GLOBAL | 8 | Mix of SvalSat, KSAT, NASA, ISRO, ESA, McMurdo |
| NONE | 0 | Empty network (no contacts) |

### Functions

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| get_ground_stations() | (none) | Returns active station list. Each station: {name, lat, lon, alt_m, min_elevation_deg, country}. |
| set_ground_stations(network=None, stations=None) | str¦None, list¦None | Sets by preset name or custom list. Returns {status, network, count}. |
| add_custom_station(name, lat, lon, alt_m=0, min_elevation_deg=5) | str, float, float, float, float | Adds single station. Sets network to CUSTOM. |
| remove_station(name) | str | Removes by name. Returns {status, removed, count}. |
| check_contact_now(position_eci, current_time) | list[3], datetime | Checks visibility to all stations. Returns {in_contact, station, elevation_deg} for best station. |

### GroundStationPassPredictor Class

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| __init__(min_duration_sec=30.0) | float | Minimum pass duration threshold. |
| compute_passes(mission_state, duration_hours=24.0) | MissionState, float | Propagates orbit at 10-second intervals. For each station, detects AOS/LOS transitions. Returns sorted list of pass dicts: {station_name, latitude, longitude, country, aos_time, los_time, duration_sec, max_elevation_deg}. |

## 4.8 autonomy_manager.py — Deterministic Autonomy

File: backend/core/autonomy_manager.py. Class: AutonomyManager. Three-mode deterministic state machine with hysteresis, configurable objective rules, subsystem coupling, feasibility validation, and temporal awareness. Every decision is fully traceable.

### Modes & Transitions

| Mode | Enter Threshold | Exit Threshold | Allowed Objectives |
| --- | --- | --- | --- |
| AUTONOMOUS | risk < 0.2 (from GUARDED) | risk >= 0.3 | All objectives |
| GUARDED | risk >= 0.3 | risk < 0.2 (to AUTO) or >= 0.6 (to SAFE) | Safe objectives only |
| SAFE | risk >= 0.6 or has_critical | risk < 0.5 | Recovery/maintenance only |

Escalation (toward SAFE) is always immediate. De-escalation requires 30-second minimum dwell time to prevent oscillation.

### Confidence Computation (4 factors)

- Factor 1 (weight 0.4): Risk score impact — score -= risk_score * 0.4

- Factor 2 (weight 0.2): Violation count — >=3 violations: -0.2; >=1: -0.1

- Factor 3 (weight 0.2): Critical violations — each critical: -0.1 (max -0.2)

- Factor 4 (weight 0.2): Margin health — >=3 unhealthy margins: -0.2; >=1: -0.1

Levels: HIGH (score >= 0.8), MEDIUM (>= 0.5), LOW (< 0.5).

### Subsystem Coupling Effects

| Effect Type | Condition | Recommendation |
| --- | --- | --- |
| POWER_PAYLOAD_CONFLICT | battery_soc < 40 AND storage > 70% | Prioritize downlink over imaging |
| COMMS_STORAGE_CONFLICT | link_status != NOMINAL AND storage > 80% | Defer new imaging until comms restored |
| ECLIPSE_POWER_STRESS | in_eclipse AND battery < 50% | Minimize all non-essential loads |
| THERMAL_POWER_CONFLICT | thermal WARNING/CRITICAL AND battery < 50% | Assess if battery can sustain heater |

### Objective Rules (configurable, 7 defaults)

| Objective | Condition | Action | Allowed Modes |
| --- | --- | --- | --- |
| Power Recovery | battery_soc < 30 | defer_payload | All |
| Comms Recovery | link_status != NOMINAL | repoint_antenna | All |
| Data Offload | storage_pct > 90 | prioritize_downlink | AUTO, GUARDED |
| Thermal Protection | component_temp > 75 | reduce_solar_exposure | All |
| Target Tracking | has_upcoming_tasks = true | slew_to_target | AUTO, GUARDED |
| Eclipse Conservation | solar_current < 0.2 | reduce_nonessential_loads | All |
| Nominal | (default, no conditions) | maintain_nominal | All |

### Key Methods

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| evaluate(telemetry, constraint_result, upcoming_tasks, predictions) | dict, dict¦None, list¦None, dict¦None | Main entry. Computes confidence, coupling, feasibility, temporal context, selects objective, transitions mode. Returns get_status() dict. |
| _compute_confidence(risk, violations, margins, telemetry) | float, list, dict, dict | Multi-factor confidence. Returns (level_str, score_float). |
| _evaluate_coupling(telemetry, margins) | dict, dict | Returns list of coupling effect dicts. |
| _evaluate_feasibility(telemetry, margins, predictions) | dict, dict, dict¦None | Checks: power (min 20% SOC), communication (min 5 dB SNR), storage (max 95%), attitude (max 2x slew rate), power_margin (net > payload/1.2). Returns dict of checks. |
| _evaluate_temporal(telemetry, predictions, upcoming_tasks) | dict, dict¦None, list¦None | Returns {eclipse_imminent, contact_imminent, task_imminent, power_trend, upcoming_events}. |
| _select_objective(telemetry, violations, upcoming_tasks, feasibility, coupling, temporal) | ... | Priority-ordered rule evaluation with AND logic. Returns (name, trigger_constraints, rejected_list). |
| _transition_mode(risk_score, recommended_mode, now) | float, str¦None, datetime | Hysteresis: separate enter/exit thresholds. Escalation immediate, de-escalation requires 30s dwell. |
| set_mode(mode, operator, reason) | str, str, str | Operator override. Returns {status, mode, override_active, operator, previous_mode}. |
| release_override(operator) | str | Resume automatic control. Returns {status, override_active, override_duration_sec, previous_mode}. |
| get_status() | (none) | Returns full status dict: mode, current_objective, risk_score, confidence_level/score, last_decision, override_info, pending_transition, feasibility, coupling_effects, temporal_context. |
| get_decisions_log() / get_latest_decisions(count=10) | (none) / int | Returns full or latest N decision objects from deque(maxlen=100). |

## 4.9 command_engine.py — Telecommand Generation

File: backend/core/command_engine.py. Class: CommandEngine. Converts scheduled tasks into telecommand sequences with operator approval workflow.

### Command Templates (per task type, 5 commands each)

| Task Type | Command Sequence |
| --- | --- |
| IMAGING | ADCS_SLEW -> PAYLOAD_ON -> IMAGING_START -> IMAGING_STOP -> PAYLOAD_OFF |
| DOWNLINK | ADCS_SLEW -> COMMS_TX_ON -> DOWNLINK_START -> DOWNLINK_STOP -> COMMS_TX_OFF |
| MANOEUVRE | ADCS_SLEW -> THRUSTER_ARM -> BURN_START -> BURN_STOP -> THRUSTER_SAFE |
| CONTACT | ADCS_SLEW -> COMMS_TX_ON -> CONTACT_START -> CONTACT_STOP -> COMMS_TX_OFF |

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| generate_sequence(plan_details, plan_id=None) | list[dict], str¦None | Creates SEQ-{uuid} with CMD-{uuid} entries. 5-second delays between commands. Returns sequence dict with status=PENDING. |
| approve_sequence(sequence_id, operator) | str, str | Sets status=APPROVED, approved_by, approved_at. All commands set to APPROVED. |
| get_sequence(id) / get_all_sequences() / get_log() | str / () / () | Retrieval methods. get_all returns reversed OrderedDict values. |
| log_command(command, status) | str, str | Logs ad-hoc operator commands (action=ADHOC). |

## 4.10 mission_planner.py — Task Scheduling

File: backend/core/mission_planner.py. Greedy scheduler with feasibility scoring and conflict detection.

### Resource Models

| Task Type | Power (W) | Data Rate (GB/s) |
| --- | --- | --- |
| IMAGING | 15.0 | 0.5 |
| DOWNLINK | 20.0 | 0.0 |
| MANOEUVRE | 12.0 | 0.0 |
| CONTACT | 10.0 | 0.0 |
| IDLE | 1.0 | 0.0 |

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| generate_mission_plan(requests, mission_state) | list[UserRequest], MissionState¦None | Sort by priority, check for temporal overlap, schedule. Returns MissionPlan. |
| compute_feasibility(task, mission_state, passes=None) | dict, MissionState, list¦None | Composite score = 0.50*power + 0.25*contact + 0.25*sunlit. Power score: >60% -> 1.0, >40% -> 0.8, >25% -> 0.5, >10% -> 0.2, else 0. Returns {task_id, feasibility_score, risk_level (LOW¦MEDIUM¦HIGH), factors}. |
| detect_conflicts(tasks, mission_state) | list, MissionState | Detects TIME_OVERLAP, MODE_CONFLICT (IMAGING+DOWNLINK), POWER_DEFICIT (<20% projected). |
| calculate_energy_cost(mode, duration_sec) | str, float | Returns Wh consumed. |
| calculate_data_volume(mode, duration_sec) | str, float | Returns GB generated. |

## 4.11 telemetry_manager.py — WebSocket & Frames

File: backend/core/telemetry_manager.py. ConnectionManager class for WebSocket lifecycle and build_telemetry_frame() for frame construction.

### ConnectionManager Methods

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| connect(ws) | ws: WebSocket | Accepts and registers WebSocket client. |
| disconnect(ws) | ws: WebSocket | Removes client from active list. |
| broadcast(data) | data: dict | Sends JSON to all clients. Auto-disconnects on failure. |
| client_count (property) | (none) | Returns len(active_connections). |

### build_telemetry_frame(state, fdir_alerts, source)

Constructs nested frame dict with 7 subsystem groups (position, velocity, power, thermal, comms, attitude, storage) plus 30+ flat legacy fields for backward compatibility. Source parameter: "LIVE" (during contact), "PREDICTED" (during blackout), or "BUFFERED" (replayed from buffer).

## 4.12 telemetry_recorder.py — Auto-Recording

File: backend/core/telemetry_recorder.py. Class: TelemetryRecorder. Thread-safe CSV auto-recording (starts on init) with optional JSON snapshot sessions.

CSV Columns (30): timestamp, source, latitude, longitude, altitude_km, speed_km_s, vx, vy, vz, battery_pct, battery_wh, bus_voltage, solar_panel_current_a, in_eclipse, panel_temp_c, battery_temp_c, link_status, snr_db, data_rate, nearest_station, attitude_mode, pointing_error, angular_rate, storage_used_gb, storage_pct, in_contact, contact_station, contact_elevation_deg, blackout_duration_sec, alert_count.

| Method / Function | Parameters | Returns / Description |
| --- | --- | --- |
| __init__(output_dir='telemetry_logs') | str | Creates dir, opens CSV with DictWriter, writes header, prints path. |
| record(frame, source, alerts=None) | dict, str, list¦None | Flattens frame, writes CSV row. Flushes every 60 frames. Optionally buffers to JSON. |
| start_json() / stop_json() | (none) / (none) | Start/stop JSON snapshot recording. stop_json() writes file and returns path. |
| status() | (none) | Returns {csv_file, csv_frames, json_recording, json_buffered_frames}. |

# 5. Backend API Routes — Endpoint Reference

## 5.1 core.py — System Endpoints

| Method | Endpoint | Response |
| --- | --- | --- |
| GET | / | Health check: {system, version, status, uptime_seconds, ws_clients, tle_loaded} |
| GET | /satellite-status | Full MissionState.get_state() snapshot (~40 fields) |
| POST | /reset | Hard reset all systems: satellite, FDIR, commands, autonomy. Returns new state. |

## 5.2 tle.py — TLE Endpoints

| Method | Endpoint | Description |
| --- | --- | --- |
| POST | /tle/load | Body: {norad_id: int}. Fetches TLE from CelesTrak, resets satellite state, propagates to now. Broadcasts tle_loaded event. |
| GET | /tle/current | Returns TLE metadata: norad_id, satellite_name, tle_line1, tle_line2, loaded, epoch_age_hours. |

## 5.3 flight.py — Flight Dynamics Endpoints

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /orbit/prediction | 90-min trajectory (90 points at 60s intervals). Returns {points: [{lat, lon, alt_km}]}. |
| GET | /flight/orbital-elements | Keplerian elements snapshot from current state. |
| GET | /flight/passes | 24-hour pass predictions for all active stations. |
| GET | /flight/ground-stations | Active station list + network name. |
| GET | /flight/ground-networks | Available preset networks with station counts. |
| POST | /flight/ground-stations/set | Body: {network: str} or {stations: list}. Switch network. |
| POST | /flight/ground-stations/add | Body: {name, lat, lon, alt_m, min_elevation_deg}. Add custom station. |
| POST | /flight/ground-stations/remove | Body: {name: str}. Remove station by name. |

## 5.4 fdir.py — FDIR Endpoints

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /fdir/alerts | Active alerts + full history array. |
| GET | /fdir/status | {active, rules_enabled, rules_count, active_alerts, total_alerts}. |
| GET | /fdir/summary | Risk status, critical/warning counts, auto_actions_today. |

## 5.5 planning.py — Planning & Commands

| Method | Endpoint | Description |
| --- | --- | --- |
| POST | /generate-plan | Body: {requests: [{lat, lon, priority}]}. Runs feasibility check, greedy scheduler, generates telecommand sequence. Returns plan_details, feasibility_scores, conflicts. |
| GET | /power/prediction | 90-min SOC prediction with scheduled task loads. |
| GET | /commands | All command sequences (reversed chronological). |
| GET | /commands/log | Full command log (reversed). |
| GET | /commands/{sequence_id} | Single command sequence by ID. |
| POST | /commands/{sequence_id}/approve | Approve sequence for dispatch. |
| POST | /commands/send | Ad-hoc command: SAFE MODE, PAYLOAD ON/OFF, ATTITUDE NADIR/SUN, TX HIGH/LOW, HEATER ON/OFF. |

## 5.6 intelligence.py — Intelligence Layer

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /intelligence/autonomy | Full autonomy status: mode, objective, confidence, feasibility, coupling, temporal context. |
| GET | /intelligence/constraints | Constraint violations, margins, risk_score, recommended_mode. |
| GET | /intelligence/margins | Per-subsystem margin status (6 categories). |
| GET | /intelligence/power-projection | Battery projection: current, at next eclipse, at end of orbit, warnings. |
| GET | /intelligence/decisions | Full decision log. |
| GET | /intelligence/decisions/latest?count=10 | Latest N decisions. |
| GET | /intelligence/feasibility | All feasibility checks with all_feasible flag. |
| GET | /intelligence/coupling | Active subsystem coupling effects. |
| POST | /intelligence/override | Body: {mode, operator, reason}. Set operator override. |
| POST | /intelligence/override/release | Release override, resume automatic control. |

## 5.7 websocket.py — Telemetry WebSocket

Endpoint: WS /ws/telemetry. Server-push only at 1 Hz. Client connects and receives telemetry frames. Message types: "telemetry" (live frame + alerts), "buffer_dump" (array of buffered frames on contact acquisition), "tle_loaded" (TLE change notification).

## 5.8 recorder.py — Recording Endpoints

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /recorder/status | CSV path, frame count, JSON recording state. |
| POST | /recorder/json/start | Start JSON snapshot session. |
| POST | /recorder/json/stop | Stop and save JSON snapshot to file. |
| GET | /recorder/download/csv | Download current session CSV file. |
| GET | /recorder/download/json | Download most recent JSON snapshot. |

# 6. Application Entry Point — main.py

File: backend/main.py. Initializes FastAPI app, creates all global engine instances, registers route modules, and runs the 1 Hz simulation loop.

### Global Objects

| Attribute | Type / Default | Description |
| --- | --- | --- |
| satellite | MissionState | Central digital twin |
| tle_manager | TLEManager | TLE fetch and SGP4 propagation |
| fdir_engine | FDIREngine | Fault detection and response |
| ws_manager | ConnectionManager | WebSocket broadcast manager |
| pass_predictor | GroundStationPassPredictor | 24-hour pass prediction |
| command_engine | CommandEngine | Telecommand sequence management |
| autonomy_manager | AutonomyManager | 3-mode autonomy state machine |
| telemetry_recorder | TelemetryRecorder | CSV/JSON recording |
| intelligence_cache | dict | Cached constraints + autonomy results (served by REST without recomputation) |

### Simulation Loop (telemetry_loop)

Runs as asyncio background task at 1 Hz (asyncio.sleep(1.0)). Each tick: (1) satellite.tick(1.0) to advance state, (2) check_contact_now() for ground station visibility, (3) satellite.update_contact() with contact result, (4) fdir_engine.evaluate() for fault detection, (5) evaluate_constraints() for margin analysis, (6) project_power() for predictions, (7) autonomy_manager.evaluate() for mode/objective selection, (8) if in_contact: build live frame, record, broadcast; if contact_acquired: dump buffer first; (9) if blackout: buffer telemetry, build predicted frame, record, broadcast.

### App Configuration

FastAPI with CORS middleware (allow_origins=['*']). Lifespan context manager starts the telemetry_loop task on startup and cancels it on shutdown. 8 route modules registered: core, tle, flight, fdir, planning, intelligence, websocket, recorder.

# 7. Frontend Architecture

React 19 application built with Vite 7. Key packages: CesiumJS for 3D globe, Leaflet for 2D maps, Recharts for charts, Lucide React for icons, React Router DOM for navigation.

### Components

| Component | Description |
| --- | --- |
| CesiumGlobe.jsx | 3D Earth sphere with satellite position, groundtrack polyline, orbit trail. Uses Cesium Viewer. |
| GroundTrack2D.jsx | Leaflet map with satellite marker, ground station markers, and footprint circle. |
| Telemetry.jsx | Real-time sidebar showing all subsystem data (position, power, thermal, comms, attitude, storage). |
| ControlStrip.jsx | Operator command bar for ad-hoc commands (SAFE MODE, PAYLOAD ON/OFF, etc.). |
| FDIRPanel.jsx | Alert list with severity badges, corrective actions, and risk status indicator. |
| AutonomyPanel.jsx | Displays current mode, objective, confidence level/score, and override controls. |
| SchedulePanel.jsx | Task list, timeline visualization, and command sequence approval buttons. |
| Header.jsx | Navigation tabs between dashboards. |
| LayoutManager.jsx | Multi-panel responsive layout with resizable panels. |

### Pages

| Page | Contents |
| --- | --- |
| ControlDashboard.jsx | Primary view: CesiumGlobe + GroundTrack2D + Telemetry sidebar + ControlStrip. |
| FlightDashboard.jsx | Orbital elements display, pass prediction table, power prediction charts (Recharts). |
| FDIRDashboard.jsx | Alert history timeline, anomaly analysis, risk trend charts, FDIR summary cards. |
| ScheduleDashboard.jsx | Mission planner form, task queue table, command sequence viewer, approval workflow. |

### Hooks & Services

| File | Description |
| --- | --- |
| useWebSocket.js | Custom hook: connects to WS /ws/telemetry, auto-reconnect on disconnect, provides latest frame and connection state. |
| useTheme.jsx | Dark/light mode toggle state with CSS variable switching. |
| api.js | Axios-based HTTP client wrapping all backend REST endpoints with error handling. |
| cesiumOrbitUtils.js | Helper functions for CesiumJS orbit polyline generation and entity management. |

# 8. Configuration Reference — satellite_config.json

Master configuration file at config/satellite_config.json with 11 major sections. Loaded once at startup via load_config(). All engines read from this config.

| Section | Key Parameters |
| --- | --- |
| satellite | name: DISHA-SAT, norad_id: null |
| orbit | initial_position_eci_km: [7000,0,0], initial_velocity_eci_km_s: [0,7.5,0] |
| power | battery_capacity_wh: 500, solar_array_output_w: 18, base_load_w: 3, bus_voltage_nominal_v: 12, task_loads_w: {IMAGING:8, DOWNLINK:12, MANOEUVRE:6, CONTACT:10} |
| thermal | panel_temp_nominal_c: 25, battery_temp_nominal_c: 22, panel_warning_high/low, panel_critical_high/low, battery_warning_high/low, battery_critical_high/low |
| storage | capacity_mb: 1048576 (1024 GB) |
| comms | snr_nominal_db: 15, snr_warning_db: 8, snr_critical_db: 5 |
| attitude | default_mode: NADIR, pointing_error_limit_deg: 2.0, angular_rate_limit_deg_s: 1.0 |
| fdir_rules | 11 rules: BATT_CRITICAL/LOW, TEMP_PANEL/BATT_HIGH/LOW, STORAGE_HIGH, SNR_LOW/CRITICAL, POINTING_ERROR, ALT_LOW |
| constraint_rules | 12 weighted rules across POWER, THERMAL, THERMAL_BATT, COMMS, COMMS_SNR, STORAGE, ORBIT categories |
| autonomy | mode_thresholds (safe_enter:0.6, safe_exit:0.5, guarded_enter:0.3, guarded_exit:0.2), min_dwell_time_sec:30, objective_rules (7 rules), feasibility limits, coupling weights |
| ground_stations | 8 ISRO ISTRAC stations: Bangalore, Lucknow, Sriharikota, Thiruvananthapuram, Port Blair, Mauritius, Brunei, Biak |

# 9. Key Design Patterns & Architecture Decisions

- Single Source of Truth: MissionState holds all satellite state; no data duplication across modules. All engines read from and write to this single object.

- Deterministic Autonomy: All decisions traceable to rules + constraints. No ML/AI. Every decision object includes trigger_constraints, coupling_effects, feasibility_summary, and temporal_context.

- Contact-Aware Telemetry: Live WebSocket broadcast during ground contact. Onboard buffer (5400 frames, ~90 min) during blackout. Buffer dump on contact acquisition.

- Hysteresis Control: Mode transitions use separate enter/exit thresholds with 30-second minimum dwell time. Escalation immediate, de-escalation requires dwell.

- Configurable Engines: FDIR rules (11), constraint rules (12), autonomy objectives (7) all loaded from satellite_config.json. No hardcoded policies.

- Intelligence Cache: Constraint and autonomy results computed each tick in the simulation loop and cached in intelligence_cache dict. REST endpoints serve from cache without recomputation.

- Multi-Level Abstraction: Flight dynamics (pure functions, no state), mission state (stateful digital twin), autonomy (behavioral state machine) cleanly separated.

- Legacy Compatibility: Telemetry frames include both nested subsystem dicts and flat fields. State dict includes both modern (battery_soc) and legacy (battery_pct) field names.

- Thread Safety: TelemetryRecorder uses threading.Lock for CSV/JSON operations. WebSocket broadcast auto-disconnects failed clients.